M-S Cash Drawer Corporation's USB cash drawer interface

Driver Install Instruction sheet

For Windows 95/98/Me/2000

For all drawers with the M-S USB interface

Windows 95

- Verify you have the correct version of Windows 95 installed:

- Select  Start->Settings->Control Panel

- Open  System

- Select the General tab

- Confirm you have revision "b" or higher installed:  System: 4.00.950 b

- If you do NOT have revision "b" or higher you must stop here and upgrade your version of Windows 95

2.)	Verify that you have the Windows 95 USB Supplement installed:

- In the Control Panel select Add/Remove Programs

- Search for "USB Supplement to OSR2" in the installed programs.

- If you have the supplement installed, proceed to the Windows 98 install for the M-S Cash Drawer USB driver.

- If you do NOT have the supplement installed you must install it from your original Windows 95 CD before proceeding.

(The following steps are listed for your convenience only.  If you require more detailed assistance installing or changing Windows 95 components, please contact your computer reseller or Microsoft Corporation directly.)

- Locate your original Windows 95 install CD.

- Open the CD using Explore

- Open the Other folder

- Open the USB folder

- Select and run USBSUPP.EXE

- If your supplement installation is successful, proceed to the Windows 98 install for the M-S Cash Drawer USB driver.

Windows 98/Me/2000

You must perform this initial driver install before attaching the cash drawer to your computer's USB port.

- Insert the M-S Cash Drawer USB driver install CD into your CD drive.

- Install should autorun.  If it does not, select Start->Run->e:\setup.exe (where e: is the drive letter of your CD-ROM)

- The driver files will be copied to your computer.

- Your computer will need to reboot for the drivers to be active.

After reboot:

- Make sure the driver CD is still in the drive.

- Attach the cash drawer to your computer's USB port.

- Your computer should auto-detect the cash drawer and install the final files from the CD.

To test the cash drawer:

- Select  Start->Programs->M-S Cash Drawer Controller->M-S Cash Drawer USB Controller

- Set the switches on the cash drawer to the USB ID you want (default = 1).

- You should be able to trigger and register drawer status.

| USB ID Chart |  |  |  |  |
| --- | --- | --- | --- | --- |
| USB ID | SW1 | SW2 | SW3 | SW4 |
| 1 * | 0 | 0 | 0 | 0 |
| 2 | 1 | 0 | 0 | 0 |
| 3 | 0 | 1 | 0 | 0 |
| 4 | 1 | 1 | 0 | 0 |
| 5 | 0 | 0 | 1 | 0 |
| 6 | 1 | 0 | 1 | 0 |
| 7 | 0 | 1 | 1 | 0 |
| 8 | 1 | 1 | 1 | 0 |
| * = Factory default | 0 = Off 1 = On |  |  |  |